| 물 품 규 격 서 |  |  |
| --- | --- | --- |
| 1. 품 명 | 영 문 | Low Vibration Cryogen-Free Optical Cryostat (Platform) |
|  | 국 문 | 저진동 무냉매 광학 저온기 (플랫폼) |
| 2. 규격(성능 및 사양) : Performance Specifications Platform Temperature Range : <3.2 K - 350 K Platform Vibrational Stability : <5 nm ptp (at base temp.) Cool Down Time : ~2 hours to 4.2 K Cooling Power : 130 mW @ 4.2 K Sample Chamber Dimensions : 53mm dia. x 63mm height (inside radiation shield) Environment : Sample-in-vacuum Positioning Modules : Manually adjustable positioner, XYZ Nanopositioners Adapter Modules : Exchange boss; Agile Temperature assy. (both include right-angle plate) Sample Mount Modules : Transmission, reflection, DC & RF electrical Temperature Sensors : 1 platform + 1 sample sensor included. 1 available user-channel Thermal Lagging : Two 30 K lagging points Sample Platform Platform Style : Circular mounting plate w/ 1-inch M3 bolt pattern Beam Height : 137.1 mm from table Sample Access : Lift off outer vacuum shroud and bolt-on inner radiation shield Standard I/O (Included) : 25-line DC side panel; 20-line DC on PCB in vacuum chamber, 2x 20GHz RF channels with lagged flex-coax routed to sample chamber Optical Ports : 5x 50 mm vacuum windows (4 radial + 1 top) with corresponding 30 mm , internal “cold windows” on radiation shield Window Material Standard: AR-coated Fused Silica, User-replaceable module with alternate materials available Acceptance Angle : 60° full angle sample at center of chamber Standard Working Distance : Horizontal axis: >25.3 mm / Vertical axis: >8.8 mm Low Working Distance : (Optional), Vertical axis: >3.3 mm |  |  |
| 3. 기타 - 납품조건: 현장설치도 - 납품기한: 2023년 12월 29일 - 납품장소: 원천관 136호(공동기기실) - 하자보수조건: 무상 하자보수 1년 - 기타요구사항: 검사검수 및 기기운용 교육 |  |  |