| 품 명 | 반도체특성분석장치 1식 |
| --- | --- |

| A. | Composition |  |
| --- | --- | --- |
|  | - SEMI CHARACTERIZATION SYST W/FPD, NO SMU 1EA - MEDIUM POWER SOURCE-MEASURE UNIT 2EA - REMOTE PREAMPLIFIER MODULE 1EA |  |
| B. | Feature |  |
|  | - Built-in 2 Channel with Source and Measure Unit - Built-in 1 Channel with Preamp - Independent 0.1fA measurement resolution - Support both pulsed I-V testing for devices with self-heating issues - General purpose pulsing and general purpose pulsing and pulse measurement - Pulse I-V software included provides both cable - Compensation and load-line compensation - Self-contained PC provides fast test setup, powerful data analysis - Graphing and printing, and on board mass storage of test results - Built-in stress/measure, looping, and data analysis for point-and-click, reliability testing |  |
| C. | Specification |  |
|  | - Current Measurement Range: ±100nA to ±100mA - Minimum Current Measurement Resolution: ±0.1fA - Minimum Current Measurement Accuracy: ±10fA - Voltage Measurement Range: ±200mV to ±200V - Minimum Voltage Measurement Resolution: ±1uV |  |
| D. | General |  |
|  | Temperature Range - Operating : +10C to +40C - Storage : -15C to +60C Humidity Range - Operating : +5% to 80% RH, non-condensing - Storage : +5% to 90% RH, non-condensing Accessories - Reference and User Manual on CD-ROM - 236-ILC-3 Interlock cable , 3M - Built-in GPIB Interface -Customized Software(MFC, Temperature) |  |
| E. | Remarks |  |
|  | - One year warranty after installation and correct operation. - Installation & operation will be provided by the factory trained engineer free of charge. - At least 2 times training will be provided in a year without charge - Delivery time is within 8 weeks. |  |